Version update log

| Version | Date | Edit Chapter | Modification points | Modified by |
| --- | --- | --- | --- | --- |
| V0.1​​ | 2025.06.06 | / | Original version | Liu Yuzhi |
| V 0.2 | 2025.07.17​ | 4 | Add descriptions related to local communication | Liu Yuzhi |
|  |  | 7 | 7.1 Add function code 08H to the transport layer protocol 7.8 Added the ability to read the most recent record | Liu Yuzhi |
|  |  | Appendix A | 1. Read and write data objects, delete data ID 70C1H, configure calendar type (Gregorian calendar or Persian calendar). 2. The pump control switch data ID has been changed from 4800H to 70DAH. 3. Removed 70BFH daylight saving time enable. 4.70C0H Daylight Saving Time Parameter Configuration Description Change 5. "Remaining allowable usage" is changed to "Remaining allowable usage for this period". 6. The read/write attribute of the remaining amount (remaining recharge amount) has been changed from RW to R. 8. Data IDB 070H , delete status bit: Water meter (flow meter) low power alarm, pressure acquisition abnormal, pressure exceeds upper limit, pressure exceeds lower limit. 9. Data IDB 070H , add status bit: Relay fault. 1.0 . Change the ID of the minute-by-minute water usage log cumulative data to B06BH 11. Changes to the content of the scheduled upload parameter (high-frequency scheduled) data ID B061H 12. Added recently occurred events (10 records) Data ID 70EEH 13. For 2G communication, add 210BH CSQ-GPRS, delete 0033H RSRP and 0034H SINR , and re-apply for data IDs including sub-items 0033H and 0034H. Delete 70C3H, add 70F2H data ID 14. Changes in data ID for cycle and allowed water consumption within the cycle Delete 70BCH, add 70F4H data ID Delete 70BDH, add 70F5H data ID | Liu Yuzhi |
| V 0.3 | 2025.08.26​ | 7 | 1. In data ID 70F5H , delete "Default 0xFFFFFFFFFF is unlimited". 2. Data ID 2012H, default customer APN is empty (all 0s) | Liu Yuzhi |
|  |  | 8 | 1. Delete 4 30EH 2. Update the remote firmware upgrade interaction flowchart |  |
|  | 2026.03.30 | 9 | 1.In Sections 7.1, 7.2, and 7.3, the description regarding function code 01H is changed to data reporting; the description for function code 02H is changed to data dispatching. 2. | Ding |
|  | 2026.08.03 |  | Appendix A: Data Objects |  |

Contents

1. 	Scope 	4

2. 	Objective 	4

3. 	Terms and Definitions 	4

3.1. 	Master station 	4

3.2. 	Terminal device 	4

3.3. 	Session ID 	4

3.4. 	Frame number 	4

3.5. 	Data Encryption 	4

4. 	Interaction Flow Description 	5

4.1. 	Terminal-Main Station Interaction Flow 	5

4.2. 	Periodic Reporting Interaction Process 	6

4.3. 	The main station issues read / write commands 	6

4.4. 	Connection Method 	7

5. 	Protocol Format 	7

5.1. 	Format Definition 	7

5.2. 	Control Domain Description 	7

6. 	Session handshake 	7

6.1. 	Handshake Request 	7

6.2. 	Handshake Request Response 	8

7. 	Transport Layer Protocol 	8

7.1. 	Function Code 	8

7.2. 	Periodic Reporting 	9

7.3. 	Communication Ends 	9

7.4. 	Continued from version 	10

7.5. 	Reading Data Object 	10

7.6. 	Writing Data Objects 	11

7.7. 	Read record by start time 	13

Appendix A : Data Object 	16

A.1 Periodic Reporting Data Object 	16

2 bytes: Event code 	17

1 byte: Event code subdivided into event 	17

A.3 Reading and writing data objects 	17

A.4 Water usage record data object 	19

Appendix B : Example Frame 	20

B.1 Session Handshake Example Frame 	20

B.1.1 Access Handshake Request (Terminal → Master Station) 	20

B.1.2 Access Handshake Request Response (Master Station → Terminal) 	20

B.2 Data Transmission Example Frame 	20

B.2.1 Periodic Reporting (Terminal → Main Station) 	20

B.2.1.1 Periodic reporting of the first frame 	20

B.2.1.2 Periodic Upload Resume Frames 	20

B.2.2 Writing Data Objects 	20

B.2.2.1 IP Settings (Main Station → Terminal) 	20

B.2.2.2 IP Configuration Response (Terminal → Main Station) 	20

B.2.3 Resume Download (Master Station → Terminal) (Periodic Reporting of Resume Download Response) 	20

B.2.4 Communication End Frame (Master Station → Terminal) 	20

Appendix C : Verification Algorithm 	20

Appendix D : FAQ 	21

Iran Electromagnetic Water Meter Communication Protocol

- scope

This standard specifies the transmission layer communication protocol for smart remote transmission terminals.

- Purpose

This protocol is used to ensure that the business protocols it carries are transmitted securely and completely, and to authenticate the identities of the communicating parties.

- Terms and Definitions

  - Master station​

Refers to a data collection server deployed remotely.

  - Terminal device

This refers to an smart terminal capable of communicating with a remote data acquisition server, hereinafter referred to as a terminal.

  - Session ID

After the handshake, an ID number is used to maintain the session. The data collection system can retrieve terminal information based on this ID number to facilitate subsequent communication. Session ID updates are initiated by the client, and the data collection system needs to manage session ID updates. Expired session IDs are updated by the client.

  - Frame number

Frame sequence numbers are primarily used to prevent duplicate frames during a session. Each time a new session is created, both parties start calculating from an initial value ( 0 on the system side , and a higher value on the terminal side). The receiving end needs to check the sending end's frame sequence number; the current sequence number must be greater than the previous one for it to be considered normal. If a frame sequence number decreases due to rollback, it is considered invalid. This same handling applies to all instances of frame sequence numbers in this document. The session lifetime should be designed to prevent frame sequence number flipping; if a frame sequence number flips, a new session needs to be renegotiated.

  - Data encryption

3.5.1 Security Encryption Algorithm

The encryption algorithm used is AES128 ECB . During local communication, the data field in the transport layer is first XORed with A5 before being encrypted and transmitted . During remote communication , the service data area in the transport layer is directly encrypted before transmission .

3.5.2 Encryption Padding Algorithm

The padding algorithm uses the PKCS7Padding algorithm .

PKCS7Padding : The padding principle is that if N bytes are needed to complete the message, and the message length is less than N bytes, then N bytes need to be added , which is ( N-len ) *( N-len ) bytes. If the message length is exactly a multiple of N bytes, then N decimal N bytes need to be added .

For example :

It needs to be padded with 8 bytes. The string "123" has 3 bytes, so 8-3=5 . After padding , it will be "123" + 5 bytes of 5. If the string length is exactly a multiple of 8 , then another 8 bytes of 8 need to be padded .

illustrate :

This protocol uses the AES128 ECB encryption algorithm . The padding alignment bits are 128 , or 16 bytes .

3.5.3 Key

Local communication keys can be configured via local infrared, and remote communication keys can be configured via new keys issued by the system.

3.5.4 Key Management

set only after the form responds . All subsequent data interactions should use the new key. It is recommended that the last 4 bytes of the new key issued by the system differ from the old key .

- Interaction Flow Description

  - Terminal-main station interaction process

sends a handshake request to the master station (see Chapter 6, Session Handshake ) . The terminal triggers a handshake request in the following three situations:

- The terminal is powered on or restarted;

- The terminal connects to the main station for the first time;

- In the termination frame sent by the main station, the termination reason value is 01 (re-session handshake), see section 7.3 for details .

After the handshake authentication is successful, the terminal begins to interact with business data. After the interaction ends, the main station sends an end frame or other read/write commands.

Local communication only involves data ID read /write and data ID read /write response processes.

  - Periodic reporting and interaction process (remote communication)

Note:

- If the terminal actively reports data but receives no response within 30 seconds, it will resend the data, and the process will end after 3 resends.

- After the master station sends the end frame, the terminal ends the current session, enters a power-off state, and no longer receives other commands.

  - The main site issues read/write commands

The master station can only issue read and write commands after the terminal's periodic reporting has ended.

  - Connection method

The IoT communication protocol uses TCP connections. Local communication uses infrared or RS485 interfaces .

- Protocol Format

  - Format definition

Unless otherwise specified in the protocol, big-endian format shall be used.

| Fields | length | illustrate |
| --- | --- | --- |
| Frame header (HEAD) | 1 | Fixed at 68H, frame start |
| Protocol Type (T) | 1 | 4: Transport layer protocol; 5: Session handshake protocol; others are invalid. |
| Protocol Framework Version (V) | 1 | Fixed at 0 |
| Frame length (L) | 2 | Frame length, including frame header and frame footer |
| Message Sequence Number (MID) | 1 | The downlink message is identical to the most recent uplink message, and the value increments by 1 after each transmission by the smart terminal. |
| Control Domain (C) | 1 | See section 5.2 Control Domains |
| Data domain (D) | L-10 | Different protocol types have different definitions for data fields; please refer to the corresponding descriptions in each protocol for details. |
| Checksum Field (CRC) | 2 | The verification scope extends from the "message sequence number" to the "data field". |
| Frame tail | 1 | Fixed at 16H, at the end of the frame. |

  - Control Domain Description

|  | control domain | direction | name | illustrate |
| --- | --- | --- | --- | --- |
| Bit 0 - 3 | 01H | Terminal --> Main Station | Data reporting |  |
|  | 02H | Main station --> Terminal | Data distribution |  |
|  | 05H | Main station --> Terminal | End frame | The master station notifies the terminal to end the current session, and the terminal can then go to sleep. |

- Session handshake

  - Handshake request

| Data Items |  | length | illustrate |
| --- | --- | --- | --- |
| Frame header (HEAD) |  | 1 | Fixed at 68H, frame start |
| Protocol Type (T) |  | 1 | 5: Session Handshake Protocol |
| Protocol Framework Version (V) |  | 1 | Fixed at 0 |
| Frame length (L) |  | 2 | The total frame length, including the frame header and frame footer. |
| Message Sequence Number (MID) |  | 1 |  |
| Control Domain (C) |  | 1 | 01: Handshake Request |
| Composite form number | Terminal type | 2 | reserve |
|  | Meter number length | 1 | This indicates the number of BCD digits in the meter number, ranging from 1 to 34. The default is a 12-digit meter number. |
|  | Meter No. | 17 | Maximum 34-bit BCD code. If less than 34 bits, pad with 0s. |
| Handshake information version |  | 1 | The current version is 0x01 |
| clock |  | 6 | Using BCD code, YYMMDDhhmmss |
| Communication mode |  | 1 | Fixed default value: 0x00 |
| Embedded version |  | 4 | Terminal software version |
| Business Agreement Type |  | 2 | Fixed default value: 0x0002 |
| Business Agreement Version |  | 2 | For example, V1.4 would be in BCD format (0x01 0x04). |
| Operator Information |  | 1 | 0 -- China Telecom; 1 -- China Mobile; 2 -- China Unicom; (This field is retained for overseas products) |
| Terminal Manufacturer ID |  | 2 | Fixed default value: 0 |
| Terminal Identification Number |  | 2 | reserve |
| Checksum Field (CRC) |  | 2 |  |
| Frame tail |  | 1 | Fixed at 16H, at the end of the frame. |

See Appendix B.1.1 for an example frame .

  - Handshake Request Response

| Data Items | length | illustrate |
| --- | --- | --- |
| Frame header (HEAD) | 1 | Fixed at 68H, frame start |
| Protocol Type (T) | 1 | 5: Session Handshake Protocol |
| Protocol Framework Version (V) | 1 | Fixed at 0 |
| Frame length (L) | 2 | Frame length, including frame header and frame footer |
| Message Sequence Number (MID) | 1 |  |
| Control Domain (C) | 1 | 02: Handshake Request Response |
| Composite form number | 20 |  |
| Handshake result | 2 | 0 indicates success, others are error codes. |
| Session ID | 4 | Generated by the main site, fixed value. It is recommended to use the last 9 digits of the meter number (BCD code) and convert it to HEX format. |
| Checksum Field (CRC) | 2 |  |
| Frame tail | 1 | Fixed at 16H, at the end of the frame. |

See Appendix B.1.2 for an example frame .

- Transport layer protocol

The transport layer requires a session handshake first. This section mainly describes the functions of the control and data fields of the transport layer protocol.

  - Data Field (D) Format

| Fields | length | code name | illustrate |  |
| --- | --- | --- | --- | --- |
| Data field length | 2 | L | Length of data field |  |
| control code | 1 | C | BIT7 | Request identifier. 0: Request message, 1 : Response message |
|  |  |  | BIT6 | Is there a sequel ? 0 : No sequel , 1 : Sequel available |
|  |  |  | BIT5 | Reserved, 0 |
|  |  |  | BIT0~BIT4 | Function codes (see 7.1.1 Function Code Description) |
| Request sequence number | 2 | REQID | The request sequence number is determined by the request message exchanged, and the request sequence number in the response message is the same as that in the request message. |  |
| Data domain | L | D |  |  |

- 7.1.1 Function Code Description

| Function code | illustrate |
| --- | --- |
| 01H | Data object reporting |
| 02H | Data object distribution |
| 03H | Resume download ( The master station sends this command to indicate that the terminal is allowed to resume the download ) |
| 04H | Read data object |
| 05H | Write data object |
| 07H | Read records by start time |
| 08H | Read recent records (by number of records) |

7.1.1.1 Data Object Reporting

Data object reporting is used for terminals to actively report data.

| Function code | name | Data Field Description |  |  |
| --- | --- | --- | --- | --- |
| 01H | Data object reporting (The terminal requests the 03H resume function code, and the backend only responds when a resume function is available.) | Up | Number of objects | The number of data objects contained in this frame is 1 byte. |
|  |  |  | ID of the first data object |  |
|  |  |  | Length of the first data object | 1-byte unsigned integer |
|  |  |  | Content of the first data object |  |
|  |  |  | Second data object ID |  |
|  |  |  | Length of the second data object | 1-byte unsigned integer |
|  |  |  | Content of the second data object |  |
|  |  |  | ... |  |
|  |  |  | ID of the Nth data object |  |
|  |  |  | Length of the Nth data object | 1-byte unsigned integer |
|  |  |  | Content of the Nth data object |  |
|  |  | Downward | This function code does not send downlink messages. |  |

7.1.1.2 Data Object Distribution

The system sends data objects to the terminal.

| Function code | name | Data Field Description |  |  |
| --- | --- | --- | --- | --- |
| 02H | Data object distribution (Backend request; the terminal only responds with the 03H resume function code if a resume is available.) | Downward | Number of objects | The number of data objects contained in this frame is 1 byte. |
|  |  |  | ID of the first data object |  |
|  |  |  | Content of the first data object |  |
|  |  |  | Second data object ID |  |
|  |  |  | Content of the second data object |  |
|  |  |  | ... |  |
|  |  |  | ID of the Nth data object |  |
|  |  |  | Content of the Nth data object |  |
|  |  | Up | This function code does not send an uplink message. |  |

7.1.1.3 Resume Download

| Function code | name | Data Field Description |
| --- | --- | --- |
| 03H | sequel ( Sent by the responder ) | Used to respond to commands with "following frame identifier", the data field is empty. |

7.1.1.4 Reading Data Objects

| Function code | name | Data Field Description |  |  |
| --- | --- | --- | --- | --- |
| 04H | Read data object (Backend request, terminal response) | Downward | Number of objects | Number of objects requested |
|  |  |  | ID of the first data object |  |
|  |  |  | Second data object ID |  |
|  |  |  | ... |  |
|  |  |  | ID of the Nth data object |  |
|  |  | Up | Number of objects | Number of data objects contained in this frame |
|  |  |  | ID of the first data object |  |
|  |  |  | Content of the first data object |  |
|  |  |  | Second data object ID |  |
|  |  |  | Content of the second data object |  |
|  |  |  | ... |  |
|  |  |  | ID of the Nth data object |  |
|  |  |  | Content of the Nth data object |  |

If the downlink data object contains a data object that the terminal does not recognize, then during the uplink, the number of objects is 0, and there is no data object content.

If the downlink data object contains write-only data objects and restricted access data objects, then when uplinking, the number of objects is the number of downlink objects minus the number of objects that are not supported by permissions, and the data content does not contain objects that are not supported by permissions.

7.1.1.5 Writing Data Objects

| Function code | name | Data Field Description |  |  |
| --- | --- | --- | --- | --- |
| 05H | Write data object (Backend request, terminal response) | Downward | Number of objects | Number of data objects contained in this frame |
|  |  |  | ID of the first data object |  |
|  |  |  | Content of the first data object |  |
|  |  |  | Second data object ID |  |
|  |  |  | Content of the second data object |  |
|  |  |  | ... |  |
|  |  |  | ID of the Nth data object |  |
|  |  |  | Content of the Nth data object |  |
|  |  | Up | Number of objects | Number of data objects contained in this frame |
|  |  |  | ID of the first data object |  |
|  |  |  | Modification result of the first data object | 1 byte, 0 indicates success, non-zero indicates failure (2 indicates permission mismatch). |
|  |  |  | Second data object ID |  |
|  |  |  | The result of modifying the second data object | 1 byte, 0 indicates success, non-zero indicates failure (2 indicates permission mismatch). |
|  |  |  | ... |  |
|  |  |  | ID of the Nth data object |  |
|  |  |  | The modification result of the Nth data object | 1 byte, 0 indicates success, non-zero indicates failure (2 indicates permission mismatch). |

If the downlink data object contains a data object that the terminal does not recognize, then during the uplink, the number of objects is 0, and there is no data object content.

If the downlink data object contains read-only data objects and data objects with restricted access, then when uplinking, the number of objects is the number of objects sent, and the corresponding data object modification result returns 2;

If the range of downlink data objects exceeds the terminal limit (with normal permissions), then during uplink, the number of objects will be the same as the number of downlink objects, and the data object modification result will be 1.

7.1.1.6 Read records by start time

| Function code | name | Data Field Description |  |  |
| --- | --- | --- | --- | --- |
| 07H | Read records by time (Backend request, terminal response) | Downward | Record file number (i.e., data object ID) |  |
|  |  |  | Record start time | 6-byte BCD code, format YYMMDDhhmmss |
|  |  |  | Record count limit | 1-byte unsigned integer |
|  |  | Up | Record file number |  |
|  |  |  | Number of records | The number of records contained in this frame, a 1-byte unsigned integer. |
|  |  |  | Record 1 content |  |
|  |  |  | Record 2 content |  |
|  |  |  | ... |  |
|  |  |  | Record N contents |  |

If the downlink data object contains a data object that the terminal does not recognize, then during the uplink, the terminal will retransmit the previous data packet.

If the downlink record limit exceeds the maximum number of records that the terminal can upload in a single communication, the terminal will only upload the maximum number of records for that single communication. If both parties agree that resuming the transmission is allowed, subsequent records will be uploaded in a resuming manner.

7.1.1.7 Read recent records

| Function code | name | Data Field Description |  |  |
| --- | --- | --- | --- | --- |
| 08H | Read recent records (Backend request, terminal response) | Downward | Record file number (i.e., data object ID) |  |
|  |  |  | Number of records read | 1-byte unsigned integer |
|  |  | Up | Record file number |  |
|  |  |  | Number of records | The number of records contained in this frame, a 1-byte unsigned integer. |
|  |  |  | Record 1 content |  |
|  |  |  | Record 2 content |  |
|  |  |  | ... |  |
|  |  |  | Record N contents |  |

If the downlink data object contains a data object that the terminal does not recognize, then during the uplink, the terminal will retransmit the previous data packet.

If the downlink record limit exceeds the maximum number of records that the terminal can upload in a single communication, the terminal will only upload the maximum number of records for that single communication. If both parties agree that resuming the transmission is allowed, subsequent records will be uploaded in a resuming manner.

  - Periodic reporting

Data object reporting is used for terminals to periodically and proactively report data (periodic reporting). When the business data control field's resume flag is 1, the master station responds with 03H (resume function code); otherwise, the master station responds with 02H ( communication terminated, data download ). The protocol format is as follows:

| Data Items |  | length | illustrate |
| --- | --- | --- | --- |
| Frame header (HEAD) |  | 1 | Fixed at 68H, frame start |
| Protocol Type (T) |  | 1 | 4: Transport Layer Protocols |
| Protocol Framework Version (V) |  | 1 | Fixed at 0 |
| Frame length (L) |  | 2 | Frame length, including frame header and frame footer |
| Message Sequence Number (MID) |  | 1 |  |
| Control Domain (C) |  | 1 | 01: Data Reporting |
| Session ID |  | 4 |  |
| Frame number |  | 2 |  |
| Business data | Data length | 2 | The length of data after the request sequence number in the business data |
|  | Function code | 1 | 01: Object reported, no subsequent frames; 41: Object reported, subsequent frames present. |
|  | Request sequence number | 2 | The request sequence number is determined by the request message in the interaction, and the request sequence number in the response message is the same as that in the request message, and so on. |
|  | Number of objects | 1 |  |
|  | First object ID | 2 | See Appendix A.1 and Appendix A.2 |
|  | Length of the first object | 1 |  |
|  | Content of the first object | n |  |
|  | ... |  |  |
|  | The Nth object ID | 2 |  |
|  | Length of the Nth object | 1 |  |
|  | Content of the Nth object | n |  |
| Checksum Field (CRC) |  | 2 |  |
| Frame tail |  | 1 | Fixed at 16H, at the end of the frame. |

For details of the data objects, please refer to Appendix A.1 and Appendix A.2 . This protocol is compatible with terminals including communication backpacks and water meters. The corresponding data object is selected based on whether the reporting entity is a communication backpack or a water meter . See Appendix B.2.1 for example frames .

  - Communication ended

The master station notifies the terminal to end the current session, and the terminal can then go to sleep .

| Data Items |  | length | illustrate |
| --- | --- | --- | --- |
| Frame header (HEAD) |  | 1 | Fixed at 68H, frame start |
| Protocol Type (T) |  | 1 | 4: Transport Layer Protocols |
| Protocol Framework Version (V) |  | 1 | Fixed at 0 |
| Frame length (L) |  | 2 | Frame length, including frame header and frame footer |
| Message Sequence Number (MID) |  | 1 |  |
| Control Domain (C) |  | 1 | 05: End Frame |
| Session ID |  | 4 |  |
| Frame number |  | 2 |  |
| Reason for termination |  | 1 | 00: Normal termination; 01: Re-opening the handshake. |
| Business data | Data length | 2 | The length of the data following the request sequence number in the business data is fixed at 23. |
|  | Function code | 1 | 02: Communication ends, Data download. |
|  | Request sequence number | 2 |  |
|  | Number of objects | 1 | Fixed: 1 |
|  | Object ID | 2 | Fixed: 21B8H |
|  | object content | 6 | The system clock is used by the terminal for time synchronization. It uses BCD code and is in the format YYMMDDhhmmss. |
|  |  | 14 | Reserved fields |
| Checksum Field (CRC) |  | 2 |  |
| Frame tail |  | 1 | Fixed at 16H, at the end of the frame. |

See Appendix B.2.4 for example frames.

  - sequel

The master station sends this command to indicate that the terminal is allowed to resume the download.

| Data Items |  | length | illustrate |
| --- | --- | --- | --- |
| Frame header (HEAD) |  | 1 | Fixed at 68H, frame start |
| Protocol Type (T) |  | 1 | 4: Transport Layer Protocols |
| Protocol Framework Version (V) |  | 1 | Fixed at 0 |
| Frame length (L) |  | 2 | The total frame length, including the frame header and frame footer. |
| Message Sequence Number (MID) |  | 1 |  |
| Control Domain (C) |  | 1 | 02: Data Distribution |
| Session ID |  | 4 |  |
| Frame number |  | 2 |  |
| Business data | Data length | 2 | The length of the data following the request sequence number in the business data is fixed at 0. |
|  | Function code | 1 | 03: Resume Download. The master station sends this command to indicate that the terminal is allowed to resume the download. |
|  | Request sequence number | 2 |  |
| CRC check field |  | 2 |  |
| Frame tail |  | 1 | Fixed at 16H, at the end of the frame. |

See Appendix B.2.3 for an example frame .

  - Read data object

The master station sends this command to read the contents of the terminal data object.

| Data Items |  | length | illustrate |
| --- | --- | --- | --- |
| Frame header (HEAD) |  | 1 | Fixed at 68H, frame start |
| Protocol Type (T) |  | 1 | 4: Transport Layer Protocols |
| Protocol Framework Version (V) |  | 1 | Fixed at 0 |
| Frame length (L) |  | 2 | Frame length, including frame header and frame footer |
| Message Sequence Number (MID) |  | 1 |  |
| Control Domain (C) |  | 1 | 02: Data Distribution |
| Session ID |  | 4 |  |
| Frame number |  | 2 |  |
| Business data | Data length | 2 | The length of data after the request sequence number in the business data |
|  | Function code | 1 | 04: Reading data objects |
|  | Request sequence number | 2 |  |
|  | Number of objects | 1 | Number of objects requested |
|  | First object ID | 2 | See Appendix A.3 . The terminal response content differs only in the data field, as shown in the table below. |
|  | ... | 2 |  |
|  | The Nth object ID | 2 |  |
| CRC check field |  | 2 |  |
| Frame tail |  | 1 | Fixed at 16H, at the end of the frame. |

The terminal replies with the content of the data object from the main site.

| Data Items |  | length | illustrate |
| --- | --- | --- | --- |
| Frame header (HEAD) |  | 1 | Fixed at 68H, frame start |
| Protocol Type (T) |  | 1 | 4: Transport Layer Protocols |
| Protocol Framework Version (V) |  | 1 | Fixed at 0 |
| Frame length (L) |  | 2 | The total frame length, including the frame header and frame footer. |
| Message Sequence Number (MID) |  | 1 |  |
| Control Domain (C) |  | 1 | 01: Data Reporting |
| Session ID |  | 4 |  |
| Frame number |  | 2 |  |
| Business data | Data length | 2 | The length of data after the request sequence number in the business data |
|  | Function code | 1 | 84: Read data object response |
|  | Request sequence number | 2 |  |
|  | Number of objects | 1 | Number of objects requested |
|  | First object ID | 2 | See Appendix A.3 . |
|  | Content of the first object | n |  |
|  | ... | ... |  |
|  | The Nth object ID | 2 |  |
|  | Content of the Nth object | n |  |
| Checksum Field (CRC) |  | 2 |  |
| Frame tail |  | 1 | Fixed at 16H, at the end of the frame. |

If the terminal cannot recognize the data object sent by the master station, the terminal will respond with a number of objects of 0xFF, indicating no data object content.

If the data objects sent by the master station include write-only data objects and restricted access data objects, then when the terminal responds, the number of objects is the number of objects sent minus the number of objects whose permissions do not support them, and the data content does not contain objects whose permissions do not support them.

Supported data objects are listed in Appendix A.3 .

  - Write data object

The master station sends this command to write the content of the terminal data object.

| Data Items |  | length | illustrate |
| --- | --- | --- | --- |
| Frame header (HEAD) |  | 1 | Fixed at 68H, frame start |
| Protocol Type (T) |  | 1 | 4: Transport Layer Protocols |
| Protocol Framework Version (V) |  | 1 | Fixed at 0 |
| Frame length (L) |  | 2 | Frame length, including frame header and frame footer |
| Message Sequence Number (MID) |  | 1 |  |
| Control Domain (C) |  | 1 | 02: Data Distribution |
| Session ID |  | 4 |  |
| Frame number |  | 2 |  |
| Business data | Data length | 2 | The length of data after the request sequence number in the business data |
|  | Function code | 1 | 05: Writing Data Objects |
|  | Request sequence number | 2 |  |
|  | Number of objects | 1 | Number of objects requested |
|  | First object ID | 2 | See Appendix A.3 . The terminal response content differs only in the data field, as shown in the table below. |
|  | Content of the first object | n |  |
|  | ... | ... |  |
|  | The Nth object ID | 2 |  |
|  | Content of the Nth object | n |  |
| Checksum Field (CRC) |  | 2 |  |
| Frame tail |  | 1 | Fixed at 16H, at the end of the frame. |

The terminal replies with the write result of the main site's data object.

| Data Items |  | length | illustrate |
| --- | --- | --- | --- |
| Frame header (HEAD) |  | 1 | Fixed at 68H, frame start |
| Protocol Type (T) |  | 1 | 4: Transport Layer Protocols |
| Protocol Framework Version (V) |  | 1 | Fixed at 0 |
| Frame length (L) |  | 2 | Frame length, including frame header and frame footer |
| Message Sequence Number (MID) |  | 1 |  |
| Control Domain (C) |  | 1 | 01: Data Reporting |
| Session ID |  | 4 |  |
| Frame number |  | 2 |  |
| Business data | Data length | 2 | The length of data after the request sequence number in the business data |
|  | Function code | 1 | 85: Write Data Object Response |
|  | Request sequence number | 2 |  |
|  | Number of objects | 1 | Number of data objects contained in this frame |
|  | First object ID | 2 |  |
|  | Modification result of the first object | n | 1 byte, 0 indicates success, non-zero indicates failure (2 indicates permission mismatch). |
|  | ... | ... | See Appendix A.3 |
|  | The Nth object ID | 2 |  |
|  | The modification result of the Nth object | n | 1 byte, 0 indicates success, non-zero indicates failure (2 indicates permission mismatch). |
| Checksum Field (CRC) |  | 2 |  |
| Frame tail |  | 1 | Fixed at 16H, at the end of the frame. |

If the terminal cannot recognize the data object sent by the master station, the terminal will respond with a number of objects of 0xFF, indicating no data object content.

If the data objects sent by the master station include read-only data objects and data objects with restricted access, then when the terminal responds, the number of objects is the number of objects sent, and the corresponding data object modification result is returned as 2;

If the range of data objects sent by the master station exceeds the terminal limit (with normal permissions), the number of objects in the terminal response will be the number of objects sent, and the data object modification result will be 1.

The content of the data object is shown in Appendix A.3 . An example frame is shown in Appendix B.2.2.

  - Read records by start time

The main station reads the water usage records from the terminal.

| Data Items |  | length | illustrate |
| --- | --- | --- | --- |
| Frame header (HEAD) |  | 1 | Fixed at 68H, frame start |
| Protocol Type (T) |  | 1 | 4: Transport Layer Protocols |
| Protocol Framework Version (V) |  | 1 | Fixed at 0 |
| Frame length (L) |  | 2 | The total frame length, including the frame header and frame footer. |
| Message Sequence Number (MID) |  | 1 |  |
| Control Domain (C) |  | 1 | 02: Data Distribution |
| Session ID |  | 4 |  |
| Frame number |  | 2 |  |
| Business data | Data length | 2 | The length of the data following the request sequence number in the business data is fixed at 9. |
|  | Function code | 1 | 07: Read records by start time |
|  | Request sequence number | 2 |  |
|  | Data Object ID | 2 | See Appendix A.4 |
|  | Record start time | 6 | BCD code, format YYMMDDhhmmss |
|  | Number of records | 1 | Fixed: 1 |
| CRC check field |  | 2 |  |
| Frame tail |  | 1 | Fixed at 16H, at the end of the frame. |

The terminal replies with the main station's water usage record.

| Data Items |  | length | illustrate |
| --- | --- | --- | --- |
| Frame header (HEAD) |  | 1 | Fixed at 68H, frame start |
| Protocol Type (T) |  | 1 | 4: Transport Layer Protocols |
| Protocol Framework Version (V) |  | 1 | Fixed at 0 |
| Frame length (L) |  | 2 | Frame length, including frame header and frame footer |
| Message Sequence Number (MID) |  | 1 |  |
| Control Domain (C) |  | 1 | 01: Data Reporting |
| Session ID |  | 4 |  |
| Frame number |  | 2 |  |
| business data | Data length | 2 | The length of data after the request sequence number in the business data |
|  | Function code | 1 | 87: Read record response by start time |
|  | Request sequence number | 2 |  |
|  | Data Object ID | 2 | See Appendix A.4 |
|  | Number of records | 1 | Number of records contained in this frame |
|  | Record content | n | See Appendix A.4 |
| Checksum Field (CRC) |  | 2 |  |
| Frame tail |  | 1 | Fixed at 16H, at the end of the frame. |

If the data object sent by the master station contains a data object that the terminal does not recognize, then when the terminal responds, the data object ID is the downlink data object ID, the number of records is 0xFF, and there is no record content.

The contents of the data object are shown in Appendix A.4 .

  - Read recent records

The main station reads the water usage records from the terminal.

| Data Items |  | length | illustrate |
| --- | --- | --- | --- |
| Frame header (HEAD) |  | 1 | Fixed at 68H, frame start |
| Protocol Type (T) |  | 1 | 4: Transport Layer Protocols |
| Protocol Framework Version (V) |  | 1 | Fixed at 0 |
| Frame length (L) |  | 2 | Frame length, including frame header and frame footer |
| Message Sequence Number (MID) |  | 1 |  |
| Control Domain (C) |  | 1 | 02: Data Distribution |
| Session ID |  | 4 |  |
| Frame number |  | 2 |  |
| Business data | Data length | 2 | The length of the data following the request sequence number in the business data is fixed at 9. |
|  | Function code | 1 | 08 : Read recent records |
|  | Request sequence number | 2 |  |
|  | Data Object ID | 2 | See Appendix A.4 |
|  | Number of records | 1 | 0 - 255 |
| CRC check field |  | 2 |  |
| Frame tail |  | 1 | Fixed at 16H, at the end of the frame. |

The terminal replies with the main station's water usage record.

| Data Items |  | length | illustrate |
| --- | --- | --- | --- |
| Frame header (HEAD) |  | 1 | Fixed at 68H, frame start |
| Protocol Type (T) |  | 1 | 4: Transport Layer Protocols |
| Protocol Framework Version (V) |  | 1 | Fixed at 0 |
| Frame length (L) |  | 2 | Frame length, including frame header and frame footer |
| Message Sequence Number (MID) |  | 1 |  |
| Control Domain (C) |  | 1 | 01: Data Reporting |
| Session ID |  | 4 |  |
| Frame number |  | 2 |  |
| business data | Data length | 2 | The length of data after the request sequence number in the business data |
|  | Function code | 1 | 8.8 : Read recent records |
|  | Request sequence number | 2 |  |
|  | Data Object ID | 2 | See Appendix A.4 |
|  | Number of records | 1 | Number of records contained in this frame |
|  | Record content | n | See Appendix A.4 |
| Checksum Field (CRC) |  | 2 |  |
| Frame tail |  | 1 | Fixed at 16H, at the end of the frame. |

If the downlink data object contains data objects that the terminal does not recognize, then during uplink, the number of objects is the number of downlink objects minus the number of unrecognized objects, and the data content does not contain unrecognized objects.

If the downlink record limit exceeds the maximum number of records that the terminal can upload in a single communication, the terminal will only upload the maximum number of records for that single communication. If both parties agree that resuming the transmission is allowed, subsequent records will be uploaded in a resuming manner.

8. Remote firmware upgrade

The firmware upgrade process is shown in the figure :

| ID | name | use |
| --- | --- | --- |
| 430CH | Server upgrade request | Deployed when a local upgrade is initiated. Includes the upgrade request, current firmware version, and target firmware version. |
| 4305H | Firmware information | Deployed when local upgrade is initiated. Includes firmware version, size, and CRC checksum. Note: Security protocols allow multiple IDs to be deployed simultaneously. |
| 4306H | Firmware segmentation parameters | The terminal requests a sub-firmware package. This includes the sub-firmware package offset and length. |
| 4307H | Firmware data structure | Based on the terminal's request, a sub-firmware package is sent, containing the sub-firmware package offset, length, firmware data content, and CRC checksum. |
| 4304H | Upgrade status | The terminal returns the firmware download status. |

Appendix A: Data Objects

A.1 Periodic Reporting Data Object

The data object IDs that the terminal needs to report are shown in the table below, where the alarm status word B070H is only reported when the status changes.

| Object ID | name | length | Sub-item length | Sub-item description |
| --- | --- | --- | --- | --- |
| B064H | Terminal communication identification code | 6 | 6 | Terminal communication identification code, take the last 6 bytes of the meter number, i.e., the 12-bit BCD code meter number. |
| 70F2 H | Terminal real-time data (Only the first frame is reported) | 50 | 6 | Terminal clock, BCD code, YYMMDDhhmmss |
|  |  |  | 2 | Main voltage, unsigned integer, unit 0.001V |
|  |  |  | 2 | Backup power voltage, unsigned integer (HEX), unit 0.001V |
|  |  |  | 1 | CSQ-GPRS |
|  |  |  | 5 | Positive cumulative amount, unit 10L, unsigned integer. Current positive cumulative amount on the water meter. |
|  |  |  | 5 | Reverse cumulative amount, unit 10L, unsigned integer. Current reverse cumulative amount on the water meter. |
|  |  |  | 3 | Instantaneous flow rate, in units of 10 L/h , signed integer. |
|  |  |  | 5 | Remaining amount (remaining recharge amount) Delete This period's allowable usage volume ，unit is 10 liters, unsigned integers. |
|  |  |  | 4 | Total pump operating time , in hours, unsigned integer. |
|  |  |  | 5 | Remaining allowance for this period, in units of 10L, unsigned integer. |
|  |  |  | 5 | Additional usage for this period, in units of 10L , unsigned integer. |
|  |  |  | 3 | in the past 24 hours , maximum daily traffic The unit is 10L/h, for example: 16 means 160L/h. |
|  |  |  | 4 | Average daily flow rate, in L/h. For example, 1600 represents 16.00 L/h. 0x7FFFFFFF indicates an anomaly. |
| B05CH | Daily frozen data (Only the first frame is reported) | 10 | 5 | Daily frozen positive cumulative amount, unit 10L, unsigned integer. The cumulative positive flow rate frozen at 00:00 on the current day. |
|  |  |  | 5 | Daily frozen reverse cumulative flow, unit 10L, unsigned integer. The reverse cumulative flow rate frozen at 00:00 on the current day. |
| B070H | Terminal alarm status word (Reporting only for the first frame and when the state changes) | 3 | 1 | bit0: 0 - Normal (on), 1 - Pump off/off switch status bit 1 : 0 - Normal, 1 - Meter removed bit 2 : 0 - normal, 1 - strong magnetic interference bit 3 : 0 - Normal, 1 - Relay fault bit 4 : Reserved, 0 bit 5 : Reserved, 0, bit 6 : Reserved, 0 bit7: Reserved, 0 |
|  |  |  | 1 | bit0: 0 - Normal, 1 - Water meter (flow meter) empty pipe alarm bit1: 0 - Normal, 1 - Water meter (flow meter) excitation alarm bit2: 0 - reserved, 0 bit3: 0 - Normal, 1 - Low signal strength of water meter (flow meter) bit4: 0 - Normal, 1 - Water meter (flow meter) measurement error bit5: 0 - Normal, 1 - Water meter (flow meter) backflow alarm bit6: 0 - Normal, 1 - Water meter (flow meter) abnormally high flow alarm bit7: Reserved, 0 |
|  |  |  | 1 | bit0: 0 - normal, 1 - storage error bit1: 0 - Normal, 1 - Traffic collection error bit2: 0 - reserved, 0 bit3: 0 - Normal, 1 - Low battery alarm bit4: 0 - Normal, 1 - Power lockout alarm bit5: 0 - reserved, 0 bit6: 0 - reserved, 0 bit7: 0 - Normal, 1 - External power connection indicator |
| 70EEH | Recent events (10 items) | 90 | 9 | 6-byte time : YYMMDDhhmmss 2 bytes: Event code 1 byte: Event code subdivided into events |

A.3 Reading and writing data objects

| Object ID | name | Read and write permissions | length | illustrate |
| --- | --- | --- | --- | --- |
| 0002H | clock | R W | 6 | BCD code integer, Y YMMDDhhmmss |
| 4211 H | Main controller version number | R | 4 | BCD code |
| 0 067H | Configure customer key | W | 1 6 | Unsigned integers (HEX) |
| 7 0B6H | Valve/pump action configuration when the event occurs | R W | 2 | 2-byte configuration bits, each bit is: 0 - pump not turned off, 1 - pump turned off Bit0: Open the isolation door; default is 0, so the pump is not turned off. Bit 1: When water usage exceeds the maximum allowable amount, the pump will be shut off by default. Bit2: Disconnect the water meter power cable; default 1 shuts off the pump. Bit3: Disconnect the pump control cable; default is 1, which turns off the pump.Delete Bit4: Insufficient balance, default is 0 and pump will not turn off.. Delete Bit5: Disconnect the meter, default 1 pump off. Bit6: Strong magnetic interference, pump off by default 1. Bit7:The current cycle ends without a subsequent cycle. By default, 1 indicates that the pump is turned off. |
| 7 0F4H | Cycle start/end | RW | twenty four | Start/ end time of the four cycles First cycle begins: MMDDHH First cycle ended: MMDDHH ... Fourth cycle begins: MMDDHH End of fourth cycle: MMDDHH When both the start and end times of a cycle are set to "000000", it indicates that the cycle is invalid. |
| 70F5H | Permissible usage within the cycle | RW | 20​ | Water consumption allowed for the first cycle: 5 bytes, unsigned integer Water consumption allowed for the second cycle: 5 bytes, unsigned integer Water consumption allowed for the third cycle: 5 bytes, unsigned integer Allowable water usage for the fourth cycle: 5 bytes, unsigned integer The unit is 10L, for example, 270F is 99990L. |
| 70BEH | Production date | R | 3 | BCD code integer, Y YMMDD |
| B055H | Freeze interval (minutes) | RW | 1 | Values are limited to 30, 60 , 90, and 120 , in minutes, with 60 as the default. |
| A102H | Meter number parameters (production) (Used for the whole machine serial number) | R | 1 | Meter number length: If the highest bit is 1, it indicates that the meter number format is ASCII string, and the meter number length is 1~17; if the highest bit is 0, it indicates BCD, and the meter number length is 1~34; the default is BCD. The complete meter number is 7 bytes: 00000000 (serial number) 09 (customer code) 01 (electromagnetic water meter) 03 (using the Iranian calendar year, this year is Iranian calendar year 1403) The water meter serial number is 4 bytes long (00000000-99999999). |
|  |  |  | 1 7 | The meter number content defaults to BCD, 34 bits, padded with zeros. |
| B061H | Scheduled parameter upload (high-frequency scheduled upload) | RW | 3 | 1-byte timed upload interval, unsigned integer, unit 10 minutes, default value 72, data range: 6, 12, 18, 24, 36, 48, 72, 144 (i.e., set to 1 hour, 2 hours, 3 hours, 4 hours, 6 hours, 8 hours, 12 hours, 24 hours) 2-byte timed upload start time, BCD code, hhmm |
| 7 0C0H | Daylight Saving Time Parameter Configuration | R W | 7 | 3-byte BCD code, Day, and Hour of Daylight Saving Time Start: MMDDHH 3-byte BCD code, Day/Time of Daylight Saving Time End: MMDDHH 1-byte HEX code , daylight saving time adjustment, in 10-minute units. Units are 10 minutes, range is [-12, 12]. Setting the time to 0 indicates that daylight saving time is not activated. |
| 70DAH | Pump control | R W | 1 | 0 - Exit lock, 1 - Lock open, 2 - Lock closed, other values are invalid. |
| 200EH | staggered interval time (Peak shifting coefficient) | RW | 2 | The default value is 6. The data range is 0 to 10, where 0 indicates no peak shifting. |
| 0016H | I MEI | R | 1 5 | ASCII strings |
| A013H | I MSI | R | 1 5 | ASCII strings |
| 200AH | SIM card information ( ICCID ) | R | 20​ | ASCII strings, 20 bytes, corresponding to the SIM card's ICCID |
| 2012H | Customer A PN | RW | 3 2 | ASCII strings, If the value is less than 0, add 0s to the end. The customer's APN is empty (all 0s). |
| 2007H | Client server IPv4 address and port | RW | 1 6 | ASCII string, address: padded with 0s if less than 16 bytes |
|  |  |  | 2 | Unsigned integer, port |

A.4 Water usage record data object

| Object ID | name | length | illustrate |
| --- | --- | --- | --- |
| B06BH | Minute-by-minute water usage log cumulative record | 1 7 | 5- byte year-month-day code, BCD , Y YMMDDHHMM 8-byte positive accumulator, 8-byte negative accumulator, unsigned integer, unit 10L 3- byte instantaneous flow rate , signed integer, unit 10L/h 1 byte reserved |
| B033H | Daily water usage log positive cumulative record | 250 | 2-byte year and month, BCD code, YYMM A 248-byte positive cumulative amount, unsigned integer, containing the positive cumulative amount frozen at 24:00 every day of the specified month. Each cumulative amount is 8 bytes, in units of L. If there are no records, fill with 0. |
| B034H | Daily water usage log reverse cumulative record | 250 | 2-byte year and month, BCD code, YYMM 248-byte reverse accumulator, unsigned integer, containing the reverse accumulator frozen at 24:00 every day of the specified month. Each accumulator is 8 bytes, in L units. No records are filled with 0. |
| B035H | Monthly water usage log positive cumulative amount record | 97 | 1 byte year, BCD code, YY A 96-byte positive accumulator, unsigned integer, containing the positive accumulator frozen at 24:00 at the end of each month of the specified year. Each accumulator is 8 bytes, in L units. No records are filled with 0. |
| B036H | Monthly water usage log reverse cumulative record | 97 | 1 byte year, BCD code, YY A 96-byte reverse accumulator, unsigned integer, containing the reverse accumulator frozen at 24:00 at the end of each month of the specified year. Each accumulator is 8 bytes, in L units. No records are filled with 0. |

Appendix B: Example Frame

Data interaction use case key : Default key

B.1 Example frame of remote communication session handshake

B.1.1 Access Handshake Request (Terminal → Master Station)

680500003A0101 7EDFFC051800D1BA253D8199ABB21EDC11CD2B2D8E03FBB8F45B7815CA66C7077E28404F0B9FA99503CEE6EE31696BDA E34416

Data Domain Decryption:

09020C00000000000100000000000000000000010001010007290002605301000201050000000600

B.1.2 Access Handshake Request Response (Master Station → Terminal)

680500002A0102 7EDFFC051800D1BA253D8199ABB21EDC7A843C9425A40A5744777D82BA7EAA7B A02116

Data Domain Decryption:

09020C00000000000100000000000000000000000006001F438

B.2 Example Frame for Remote Communication Data Transmission

B.2.1 Periodic Reporting (Terminal → Main Station)

B.2.1.1 Periodic reporting of the first frame

68040000CA0201 BA516C55E4F9304C324FF43C053614051BF841F4843F7B885E050FDABDC79E4835AC818AA83B8901C7307612BAF02263 251E6AC8BA6408EEE4F5D7BA6F5E332F351CBFCFF13B150D92FEF3411EEF105EEAE2854CFB8EF5C48A3BA5C7FA919BA5 E46ECE255A8A7F5A3414AE8B1AE03F89286A7970E585C58F31FE10A17CBF2F218168B7A12010D96F8AC159AD931AD0F3 E7B0D6B271E8404826DFFFD458EC175CD6CE70E0FFC61F0BA142CC1E9B8FC78F5ED9AA7BF6746AA68C97995CA7C85C6C 209516

Data Domain Decryption:

6001F438000100AE01000005B0640600000000000170F23100010100073300000000130000000000000000000000 0000FFFFFFFF0000000000000000000000000000000000000000B05C0A00000000000000000000B070038007057 0EE5A000101000200016000000101000001017000000101000001178500000101000000054A00000101000000054 C00000101000000054C00000101000000054C00000101000000054C000001010000001F4E00031011200000054E01

B.2.2 Reading Data Objects

B.2.2.1 Time Reading ( Main Station → Terminal )

680400001A0202 CA4E10177BFD8498F7A5A94EAA9653B7 4F7016

Data Domain Decryption:

6001F43800010003040000010002

B.2.2.2 Time Read Response ( Terminal → Main Station )

680400002A0301 734D68A9EC1F4148B6FF5009270F4A237ED4152DB3D523D2E6EB2BC66BBFBE9B 1F5B16

Data Domain Decryption:

6001F43800020009840000010002000101000846

B.2.4 Communication End Frame (Master Station → Terminal)

161C16​​

Data Domain Decryption:

6001F43800040000170200000121B82509010934220000000100000000000100000001

B.3 Local Communication Example :

Data interaction use case key : Default key

B.3.1 Time Reading

|  | Raw data | After data field decryption | Data XORed with A5 |
| --- | --- | --- | --- |
| Downward | 680400001A0002 EE76F4DA083973E28E87260CDD862FCB 4AB616 | 3C3C3C3CA5C1A5A6A1A5A5A4A5A7 | 9999999900640003040000010002 |
| Up | 680400002A0001 BF2D4FD99B324E5BDE03BE2E108865D7F5C507C0F118D9C43325572F4C4AD6FB 8E7316 | 3C3C3C3CA5A4A5AC21A5A5A4A5A7A6B5B485A581 | 9999999900010009840000010002031011200024 |

B.3.2 Time Write

|  |  | After data field decryption | Data XORed with A5 |
| --- | --- | --- | --- |
| Downward | 680400002A0002 D6BB4C3976DFADE669BDE5C4D3268DBD20F03EAF4299994EFA84A164D468B3C7 398B16 | 3C3C3C3CA5C0A5ACA0A5A5A4A5A7A6B5B485A5A5 | 9999999900650009050000010002031011200000 |
| Up | 680400001A0001 F63A256EA9EC31FCC2BC325AD6E413A7 F8D716 | 3C3C3C3CA5A7A5A120A5A5A4A5A7A5 | 999999990002000485000001000200 |

B.4 Example of remote communication firmware upgrade (after the first frame is uploaded):

Data interaction use case key : goldcard00250253

|  |  | Raw data | Data field plaintext |
| --- | --- | --- | --- |
| Downward | The upgrade request ID is: 430C | 680400002A0202 BB09B702A689EB9C11CC392CFFE7AD035C6F543A65342F8E7AF47EF0E5A10335 ACEC16 | 9FD1A7930001000C02000001430C010260530202605301 |
| Up | The device upgrade status ID is: 4304 | 680400002A0301 11FD9AB856F2570A134F1B74E32857B4ABF31D22577BCC83AE8C64A776B9C87D 4D7E16 | 9FD1A793000200080100000143040401000000 |
| Downward | Firmware information: 4305 | 680400002A0302 65E79BD568196B0D35CB9EE68ED692D74610A3FAD868BFFE89AF7609A02CFA27 BBD216 | 9FD1A7930002000F0200000143050260530200000F00EE2FCC70 |
| Up | Firmware segmentation parameter ID is: 4306 | 680400003A0401 05B0E39E6C928EA28229487839CD7D0DE0E522D1A43BA844B5B88930644089E157B119C0F0DD789D2EB18F5410775136 A6BA16 | 9FD1A7930003001B0100000243050C0260530200000F00EE2FCC704306080000000000000100 |
| Downward | Firmware package data structure: 4307 | 680400012A0402 F378936EC2C91DF01EB62269DDD25C6B581F81D6D063FE86373110024D8CD148076F39C03C47676CCD78167F8326EC08E1518C727E2ED94691D8DE161991DD2F3568F73BA8C4C856F57CB1FD6A30FFA3BDB21F8CACC663D6A280E87C002F1862BDB21F8CACC663D6A280E87C002F1862BDB21F8CACC663D6A280E87C002F1862BDB21F8CACC663D6A280E87C002F186291D6AA20E214DC03B5D28FCCE09828A400985551993756034BD431D023EAA671AD525722EF7A4A6C17AD0881AC42EEE6541BE312674159408217337883D297B5B75ACF37A8AEC1421C8EEDA7F68F682195FF81FC0CFF3B4EACCFFD3BB6F02A0464F539CE4A281F21B40B3DC149F7C75207F3F5D22C6FC609FF09F74C3DB9BA15BFB4E0C18EC05375B821EDA12EE56124 E90616 | 9FD1A7930003010D0200000143070000000000000100 AAAAAAAAA026053029290DB8C00028A6B5F58209300000E27FFFFFFFFFFFFFFFF04ABA91444806873D08417F007F007F007F007F007F007F007F007F007F007F00 7F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F00 7F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007 F007F001D8080CF0BC9803F803F8012C04067801FC01FC01FC01FC01FC01FC01FC01FC01FC01FC01FC012E0280FE00FE00C901009C007F007F00198080C30DF38 03F803F801BC04062801FC01FC01FC01FC01FC01FC01FC01FC01FC01FC004602030C3B7A45261308141C3126452E2746BD6E37160B0E036E00FE00FE00FE0276A |
| Ascending | Fixed case number ID: 4306 | 680400003A0501 78A56D5B6C8EEE24E5B452D7EE344ABFE0E522D1A43BA844B5B88930644089E10BC7D1BBB4748BC52F76DBC437AEBBA6 13D416 | 9FD1A7930004001B0100000243050C0260530200000F00EE2FCC704306080000010000000100 |
| descending | Firmware package data structure: 4307 | 680400012A0502 6FC6D3FAF3AB42A1446AC67BAF1A49BB13BC47150CF9707E40237CBC0472DB8343C8D7CD72020B4479C8CD68A4A465D9D8A34064C932CC6ECF443F6268074152D582B90B95A98A0D05F00449C3185A404BD45C05AA92197D767F0D69BF7D687ED94959561878354A4DA01FEFED56E3DACA5ECD344A4FE4FC9D2FC52EFF34E232EF652DB02AB0A1DEAC7FE40AF1E303BB87DFA04C090A83F4B7F3DAA6DCA289EFF320D47CBC7417C8F7E65D5C4EEC92C7F320D47CBC7417C8F7E65D5C4EEC92C7F320D47CBC7417C8F7E65D5C4EEC92C7F320D47CBC7417C8F7E65D5C4EEC92C7628BE716DBA55C1A6002E8F12EDA2A8B81A411A2CF8F244B10826B13346D5BCEEE2C97C8660086E76E9082C6A17FF10CF11722C512CA0D0EB0452930C7E53791 EAB716 | 9FD1A7930005010D02000001430700000100000001000FE00A301018A0B8D00C743A736E977897E8B018F002F28891F8D2597338F05870B8B43B3007F007F007F007F002D808058003FBCE1916693CE9B628027A57F803F803F803F803F803F803FC04072987FC01FC01FC001602031C130E071600FE00FE00FE0027010186007F58B9078762FF591E007F5B365D150C9933B70AA73BCB007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F007F006F80818601C7819483C89D130104A148A84D198724C88804934999D0390B258C3E0D060B0E483A0AA291160B8E171FCB83500021F021 |
| Up | The firmware segmentation parameter ID is: 4306 (requesting the last frame). | 680400003A1202​​ | 9FD1A7930011001B0100000243050C0260530200000F00EE2FCC7043060800000E0000000100 |
| Downward | Firmware package data structure: 4307 | 680400012A1202 7103A1D6D9C37DE7876D2DA143D162B98BFE52076D367B0FF13CAB4C2F07740AC5DA6CDFBBF2C98C62E98E94BA3C588F944BD4F8E4BB5B30D810A08C1CD3A408EEA98696946C35ECEB5B26F0229812FD3600B5F53E6E339C61ADAA52A8C1726CFD76E11E69F1A5F1F4481704D51F5467FD76E11E69F1A5F1F4481704D51F5467FD76E11E69F1A5F1F4481704D51F5467FD76E11E69F1A5F1F4481704D51F5467FD76E11E69F1A5F1F4481704D51F5467FD76E11E69F1A5F1F4481704D51F5467FD76E11E69F1A5F1F4481704D51F5467FD76E11E69F1A5F1F4481704D51F5467FD76E11E69F1A5F1F4481704D51F5467FD76E11E69F1A5F1F4481704D51F5467FD76E11E69F1A5F1F4481704D51F5467E26F6B7782741ECEDEFA1D239FC26050 381A16 | 9FD1A793001B010D02000001430700000E000000010003F803F803F803F803F803F802FC04061801FD3E7D46A452341AA476DCDEDC1B15FEED17844A44168C66DC5EDD6E6C061C01FC01FC01E602030C00FE00FE00FE00FE0057EE8400FFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFFF7B1F |
| 上行 | 表端升级状态ID为：4304 | 680400002A1301 E6BF73DF0FE6BE094B6759FAFDD9AE2EABF31D22577BCC83AE8C64A776B9C87D AB4816 | 9FD1A793001200080100000143040403000000 |
| 下行 | 结束帧：21B8 | 680400002A150569CE8F5FED79B976E29A51BA0AEE417AE50D234B6DA5181A910045984C3A6A5335AF16 | 9FD1A79300200000110200000121B80000000100000000000100000001 |

附录C：校验算法

unsigned short CRC16(unsigned char *puchMsg, unsigned int usDataLen)

{

unsigned short wCRCin = 0x0000;

unsigned short wCPoly = 0x1021;

unsigned char wChar = 0;

while (usDataLen--)

{

wChar = *(puchMsg++);

wCRCin ^= (wChar << 8);

for(int i = 0;i < 8;i++)

{

if(wCRCin & 0x8000)

wCRCin = (wCRCin << 1) ^ wCPoly;

else

wCRCin = wCRCin << 1;

}

}

return (wCRCin);

}

Appendix D: FAQ

Q1: Is a handshake required for every report?

A1: No need. After the initial handshake, subsequent data can be transmitted using the session ID.

Q2: Will the session ID expire? What happens if it expires?

A2: The session ID validity period is related to the platform settings. After the session ID expires, the platform will not respond to terminal data. At this time, the terminal needs to actively handshake and update the data ID. If the termination reason value in the termination frame received by the terminal is 01, then a handshake request will be sent.

Q3 : The terminal resends two data frames, and the master station responds to both. How can we ensure that the data frame received by the terminal is the latest one sent by the master station ?

A3 : The downlink message and the most recent uplink message are identical via the message sequence number (MID). The MID value is incremented by 1 after each transmission by the terminal .